Publication-ready tables for Geed Deeble NDVI analysis

Table 1. Landsat image count and NDVI descriptive statistics for the Geed Deeble Forest Reserve ecosystem, MAM/Gu season, 2000–2024.

| Year | Primary sensor(s) | Image count | Mean NDVI | Min NDVI | Max NDVI | SD NDVI |
| --- | --- | --- | --- | --- | --- | --- |
| 2000 | Landsat 5 TM | 5 | 0.113 | 0.041 | 0.528 | 0.026 |
| 2008 | Landsat 7 ETM+ | 5 | 0.114 | 0.053 | 0.521 | 0.032 |
| 2016 | Landsat 8 OLI | 6 | 0.162 | 0.076 | 0.702 | 0.055 |
| 2024 | Landsat 8 OLI / Landsat 9 OLI-2 | 11 | 0.201 | -0.311 | 0.741 | 0.073 |

Note: NDVI values are unitless. Statistics were derived from seasonal Landsat Collection 2 Level-2 Surface Reflectance composites.

Table 2. NDVI vegetation class area and percentage within the Geed Deeble Forest Reserve ecosystem, 2000–2024.

| Year | NDVI range | Vegetation condition | Area (ha) | Area (%) |
| --- | --- | --- | --- | --- |
| 2000 | 0.00–0.15 | Bare or degraded land | 5,532.33 | 93.89 |
|  | 0.15–0.25 | Very sparse vegetation | 298.99 | 5.07 |
|  | 0.25–0.40 | Sparse vegetation | 32.31 | 0.55 |
|  | 0.40–0.60 | Moderate vegetation | 4.76 | 0.08 |
| 2008 | 0.00–0.15 | Bare or degraded land | 5,414.22 | 91.88 |
|  | 0.15–0.25 | Very sparse vegetation | 394.67 | 6.70 |
|  | 0.25–0.40 | Sparse vegetation | 52.09 | 0.88 |
|  | 0.40–0.60 | Moderate vegetation | 7.40 | 0.13 |
| 2016 | 0.00–0.15 | Bare or degraded land | 3,229.15 | 54.80 |
|  | 0.15–0.25 | Very sparse vegetation | 2,303.77 | 39.10 |
|  | 0.25–0.40 | Sparse vegetation | 265.51 | 4.51 |
|  | 0.40–0.60 | Moderate vegetation | 64.23 | 1.09 |
|  | > 0.60 | Dense vegetation | 5.73 | 0.10 |
| 2024 | < 0.00 | Negative / water / shadow | 0.18 | 0.00 |
|  | 0.00–0.15 | Bare or degraded land | 633.17 | 10.75 |
|  | 0.15–0.25 | Very sparse vegetation | 4,489.65 | 76.19 |
|  | 0.25–0.40 | Sparse vegetation | 548.32 | 9.31 |
|  | 0.40–0.60 | Moderate vegetation | 173.89 | 2.95 |
|  | > 0.60 | Dense vegetation | 23.18 | 0.39 |

Note: Area (%) represents the share of the AOI. Small rounding differences may occur because values are rounded to two decimal places.